# Mata Hari

## Physical Setup

Level Machine – about 1% steeper than factory recommendation

Ear Plug Tilt Hack

Verify Tilt Sensitivity – should be sensitive, but only one danger with each hard bump

Check Flipper Strength, Consider Glo Balls if playing too slow or can’t hit upper playfield

Clean/wax playfield

Clean glass

## Switch or Software Setup

3-Ball Play (S16 ON)

Disable Extra Balls, Special worth points (S30/S31 OFF)

AB Feature Conservative (S24 OFF)

Free Play if possible, else Max Credits per coin and minimum replay score

  - S28-S25 ON, S2-5 ON, S1 OFF, S10-13 ON, S9 OFF.